Develop a Python-based DNS investigation tool that accepts multiple domains from a text file rather than a single domain. Resolve the domains, organize the resulting IP addresses, identify domains sharing an IP address, and report domains for which resolution fails. The output should provide a compact reconnaissance summary suitable for preliminary investigation.

Experiment No: 44

Aim

To develop a Python-based DNS investigation tool that accepts multiple domains from a text file, resolves their IP addresses, identifies domains sharing the same IP address, and reports domains for which DNS resolution fails.

Objectives

- To understand DNS resolution.

- To resolve multiple domain names.

- To identify IP addresses associated with domains.

- To detect domains sharing the same IP address.

- To identify failed DNS resolutions.

- To generate a compact reconnaissance summary.

Theory

The Domain Name System (DNS) translates domain names into IP addresses. DNS resolution is useful during preliminary digital investigations because it can reveal the network addresses associated with domains.

When multiple domains resolve to the same IP address, they may be hosted on the same server or infrastructure. Failed resolution can also be recorded as an investigation result because it may indicate an unavailable, invalid, or non-resolving domain.

This experiment uses Python's socket module to resolve multiple domains from a text file.

Algorithm

- Start.

- Import the required Python modules.

- Read the domain list from a text file.

- Resolve each domain using DNS.

- Store the resulting IP addresses.

- Identify domains that share an IP address.

- Record domains for which resolution fails.

- Display the DNS reconnaissance summary.

- Stop.

Flowchart

Start

↓

Read Domain List

↓

Resolve Each Domain

↓

Resolution Success?

/          \

Yes           No

↓             ↓

Store IP       Record Failure

↓

Process Next Domain

↓

Group Domains by IP

↓

Identify Shared IPs

↓

Display Summary

↓

Stop

Sample Input

Create a file named domains.txt:

example.com

google.com

youtube.com

invalid-domain-example-12345.com

Run the program:

Enter domain list file: domains.txt

Sample Output

DNS RECONNAISSANCE SUMMARY

--------------------------------

Resolved Domains

--------------------------------

example.com -> 93.184.216.34

google.com -> 142.250.195.14

youtube.com -> 142.250.72.206

Domains Sharing Same IP

--------------------------------

No domains sharing the same IP were found.

DNS Resolution Failures

--------------------------------

[FAILED] : invalid-domain-example-12345.com

Actual IP addresses can vary depending on DNS configuration and when the program is executed.

Applications

- DNS investigation

- OSINT reconnaissance

- Digital forensic investigation

- Network investigation

- Domain infrastructure analysis

- Preliminary cybersecurity assessment

Advantages

- Accepts multiple domains from a single file.

- Automatically resolves domain names.

- Groups domains according to their IP addresses.

- Reports failed DNS resolutions.

- Produces a compact investigation summary.

Limitations

- DNS results can change over time.

- A shared IP address does not necessarily indicate suspicious activity.

- Some domains use multiple IP addresses.

- DNS resolution may fail because of temporary network problems.

Result

Thus, the Python program successfully performed DNS investigation on multiple domains, organized the resolved IP addresses, identified domains sharing an IP address, and reported domains for which DNS resolution failed.